Task 1

Given: Virtual address split: 4-bit VPN | 8-bit offset Physical address split: 3-bit PPN | 8-bit offset

(i)

- Number of virtual pages VPN uses 4 bits, total virtual pages = 24=16

- Bytes per page Offset length = 8 bits, page size = 28=256 Bytes

(ii) Virtual address = 0x2C8

Step 1: Separate offset (lower 8 bits) and VPN (upper bits) Offset mask = 0xFF Offset = 0x2C8&0xFF=0x08 VPN = 0x2C8>>8=0x2=2

Step 2: Look up page map VPN = 2 maps to PPN = 0x4 = 4

Result: VPN = 0x2, Offset = 0x08, Corresponding PPN = 0x4

Task 2

Page frame size = 4 KB = 4096 Bytes 8 physical page frames (0 ~ 7), total memory = 32 KB Frame address range rules: Frame N starts at N×4096, ends at (N+1)×4096−1

- Frame 0: 0 ~ 4095 → D = 4095

- Frame 2: 8192 ~ 12287 → C = 12287

- Frame 5: 20480 ~ 24575 → B = 24575

- Frame 7: 28672 ~ 32767 → A = 32767

Final values: A = 32767, B = 24575, C = 12287, D = 4095

Task 3

32-bit virtual address layout: Top 20 bits = VPN, Low 12 bits = offset Target virtual address binary: 00000000000000000011 000010110111

2.a Referenced virtual page

Extract top 20 bits: 00000000000000000011 Decimal VPN = 3

2.b Page frame value

Page table entry for VPN=3: Present bit = 1, PPN binary = 011 Binary PPN: 011 Denary (decimal) PPN: 3

2.c Construct physical address

Physical address format: PPN + original 12-bit offset

- PPN binary: 011

- Offset binary: 000010110111 Full physical address binary: 011 000010110111 Hex physical address: 0x30B7

Task 4

System parameters: Virtual memory = 232 Bytes, Physical memory = 224 Bytes Page size = 210 Bytes (1 KB), TLB capacity = 4 entries VPN = 22 bits, Offset = 10 bits, PPN = 14 bits

(i) Number of physical memory pages

Physical pages=Page sizePhysical memory size​=210224​=214=16384

(ii) Number of page table entries (total virtual pages)

Virtual pages=Page sizeVirtual memory size​=210232​=222=4194304

This equals the total entries in the full page table.

(iii) Bits per single page table entry

Each entry stores: 14-bit PPN + 3 control flags (Resident R, Dirty D, Present valid bit) Total bits per entry = 14+3=17 bits

(iv) Total memory pages occupied by the full page table

- Total bits of all page table entries = 222×17

- Convert bits to bytes: divide by 8 Total bytes=8222×17​=17×219

- Divide by page size (210 bytes per page): Page table pages=21017×219​=17×29=17×512=8704

Task 4.2 Bonus Question

Rule: Page size = 1 KB, lower 10 address bits = offset, remaining high bits = VPN

Q1: Virtual address = 0x1804

Step 1: Split VPN and offset Offset mask = 0x3FF (10 bits) Offset = 0x1804&0x3FF=0x004 VPN = 0x1804>>10=0x6=6

Step 2: MMU translation process

- MMU splits virtual address into VPN and offset.

- Search TLB for VPN = 6; no matching entry → TLB miss.

- Access main memory page table to read mapping for VPN=6.

- Page table returns PPN = 2.

- Concatenate PPN and offset to build physical address: Physical address=(2<<10)∣0x004=0x804

Q2: Virtual address = 0x0FC

Step 1: Split address Offset = 0x0FC, VPN = 0 Step 2: MMU translation process

- Split VA into VPN=0 and offset=0x0FC.

- Look up TLB: VPN=0 exists with PPN=2 → TLB hit.

- No need to access page table in memory.

- Generate physical address: Physical address=(2<<10)∣0x0FC=0x8FC

Task 5 Memory Interface

Memory cells indexed from 0 to D (14 memory locations, 16-bit data width)

Task 5.1 Address decoder analysis

- Total unique memory indexes: 14. We need enough address bits to cover all locations. 23=8<14, 24=16≥14. So 4 address input bits are required for the decoder.

- Circuit description:

- 4-bit address bus connects to a 4-to-16 binary decoder.

- Decoder outputs 14 individual select lines to each memory cell.

- Three control signals (Read, Write, Enable) feed into the decoder’s enable port to control memory operation mode.

Task 5.2 Combined address format

Address structure: 2 control bits + 3 index bits (total 5-bit address) Control bit rules:

- 01 = Read operation

- 10 = Write operation

a) Read memory location FEAD (Index B)

Index B = decimal 11, binary = 1011 Take lower 3 index bits: 011 Read control bits = 01 Full 5-bit address binary: 01 011 = 01011 Hex value = 0x0B

b) Write to memory index C

Index C = decimal 12, binary = 1100 Take lower 3 index bits: 100 Write control bits = 10 Full 5-bit address binary: 10 100 = 10100 Hex value = 0x14